SECTION 097715 – Glass Wall Surfacing

- GENERAL

  - SUMMARY

    - Section Includes:

      - Silicone acrylic hybrid glass surfacing.

    - Related Requirements:

      - 01 ## ## - [administrative requirements section]: for additional requirements of preinstallation meeting.

      - 01 ## ## - [quality section]:  for additional requirements related to Quality Requirements and testing.

      - 01 ## ## - [mockup section]:  for additional requirements related to the mockups in this section.

      - 01 ## ## - [indoor air quality section]: for additional requirements relating to indoor air quality.

      - 01 ## ## - [construction waste management section]:  Limitations on disposal of removed materials; requirements for recycling.

  - REFERENCES

    - References:

      - SMACNA (OCC) - IAQ Guidelines for Occupied Buildings Under Construction; 2007.

  - DEFINITIONS

    - Surface 1: Surface facing room.

    - Surface 2:  Surface facing wall.

  - ADMINISTRATIVE REQUIREMENTS

    - Preinstallation Meeting:  Convene _____ week before starting installation work of this section in accordance with 01 ## ## - [administrative requirements section].

      - Attendees:  Owner’s representative, Architect, Contractor, Glazing Fabricator and all Subcontractors interfacing with glass wall surfacing.

      - Review approved submittals, preparation and installation procedures and coordination and scheduling required with related work.

  - SUBMITTALS

    - Sample: Provide standard size samples for verification of selected finish for support system and each type, color, and texture of exposed finish, full thickness.

      - Glazing Size:  4 inch x 6 inch x 1/4 inch

      - Each component of exposed hardware.

    - Product Data:  For each glazing assembly including all hardware and accessories.

    - Shop Drawings:  Include manufacturer provided plans, elevations, sections, connection and attachment details, finish schedule, and locations.

      - Include furring strips, perimeter seals, metal support brackets and silicone cushion materials and joint width between glass panels.

    - Manufacturer's Installation Instructions.

    - Qualification Statements.

    - Operation and Maintenance Data:  Provide maintenance data for incorporation into operation and maintenance manuals.

    - Warranty Documentation.

  - QUALITY ASSURANCE

    - Qualifications:

      - Manufacturer Qualifications:  Company specializing in the manufacture of work specified in this section, of the quality and complexity required for this project for a minimum 10 years.  Show production facilities capable of meeting contract requirements for single-source responsibilities and warranty.

      - Installer Qualifications:  Company specializing in performing the work of this section with minimum of 3 years of experience.

    - Source Limitations: Obtain glass wall surfacing from single source.

  - DELIVERY, STORAGE, AND HANDLING

    - Comply with the manufacturer’s requirements for a warrantable installation of the installed products to meet the Performance and Design Criteria.

    - Comply with SMACNA (OCC) and Section 01 ## ## - [sustainability section].

- PRODUCTS

  - DESCRIPTION

    - Factory assembled site installed glass wall surfacing.

  - MANUFACTURERS

    - Specification is based on products listed below by ICD High Performance Coatings + Chemistries.

  - Glass wall surfacing

    - (GL-#) Silicone Acrylic Hybrid Glass Surfacing:

      - ASTM C1048, Type I, Condition C, Quality-Q3.

      - Water-Based Coating:

        - Basis of Design Coating: AquaVue.

        - Coating Color:

          - Name: [____________] Number: [____________]

          - Match paint manufacture color:  Name: [____________] Number: [____________].

        - Coating Location: Second surface.

        - Scratch Resistance:

          - 4-5H Pencil Hardness per ASTM D3363.

        - Cleaner and Water Resistance:

          - Solvent Resistance:  Passes ASTM D7835.

          - Water Soak Resistance:  Passes ICD Internal test for exposure to moisture at elevated temperatures.

      - Glass:

        - Glass Type: [Heat-strengthened] [Fully tempered]

        - Glass Clarity: [Clear] [Low Iron/Ultra clear] [Tinted].

        - Tint Color: [____________].

        - Minimum Thickness: [6 mm] [____________].

  - ACCESSORIES

    - All accessory materials required by the manufacturer for a warrantable installation of the installed products in a manner that meets the Performance and Design Criteria.

    - All accessory materials required to comply with coating manufacturers current Compatible Materials List.

    - Manufacturer's accessories required by the project:

      - Setting Blocks, spacers, and sealant:  As indicated in manufacturers installation instructions.

- EXECUTION

  - EXAMINATION

    - Verify existing conditions meet the manufacturer's requirements before starting work.

    - Verify products have been stored, and will be installed, in accordance with project's Construction Indoor Air Quality Management Plan specified in Section 01 ## ## - [indoor air quality section].

  - PREPARATION

    - Prepare surfaces to receive work in accordance with manufacturer's instructions.

  - INSTALLATION

    - General:  Install all materials in accordance with manufacturer's instructions based on conditions present.

    - All miscellaneous installation materials required to comply with EQ credit:  Low Emitting Materials, Option 1 in accordance with Section 01 ## ## - [indoor air quality section].

    - Install temporary supports as required by manufacturers installation instructions for height, area, and configuration of installation.

  - CLEANING

    - Dispose of all waste material in accordance with Section 01 ## ## - [construction waste management section] and project's Waste Management Plan.

  - PROTECTION

    - Protect installed work as required by the manufacturer to maintain product performance and design criteria.

  - SCHEDULE

    - (GWC- #):  Open office writable wall.

      - Features:

        - Color: ____________.

      - Locations:

        - ____________.

END OF SECTION